Bangkok Flood Forecast & Early-Warning System

Machine-Learning Component — Technical Documentation

Prepared for the BMA prototype sprint · July 2026

# 1. Project Overview

This project builds a road-flood forecasting and early-warning system for the Bangkok Metropolitan Administration (BMA), developed by a three-person team (Data/GIS engineering, ML engineering, Dashboard engineering) in a 15–20 day sprint. The ML component described here answers one question: given the last few hours of rain, canal-level, and flow measurements across BMA's sensor network, will each monitored road flood in the next 1, 3, and 6 hours — and how deep?

Everything below was built, evaluated on a sealed 2025 test year, and is now served through a FastAPI service that the dashboard consumes, including a live mode fed by real-time weather.

# 2. The Dataset

## 2.1 Source data

Seven years (2019–2025) of BMA telemetry at 5-minute cadence, four sensor networks, ~52 GB of raw CSV:

| Network | Stations | Role | Key columns |
| --- | --- | --- | --- |
| Flood (FL.*) | 107 | Label source + autoregressive feature | flood depth on road, cm |
| Rain (RF.*) | 131 | Primary driver | rf5min … rf24hr accumulations, mm |
| Water (WL.*) | 300 | Canal levels (leading indicator) | wl_in, wl_out01/02, m |
| Flow (FW.*) | 30 | Drainage state | flow m³/s, wl, area, velocity |

## 2.2 Data-quality findings (full-scan verified)

- Missing values are literal "NULL" strings; files carry UTF-8 BOM and CRLF endings; Rain filenames are inconsistent across years; Water station names contain commas (quoted CSV).

- No row truncation anywhere: every file ends Dec 31 23:55 with exact stations × 105,120 rows (leap years 105,408). Earlier truncation concerns were an Excel-export artifact.

- Station rosters grow over time (Flood 99→107, Water 255→300); NULL rates rise toward the present (Flood ~0%→11%, Flow 5%→16%) — the sensor network is degrading.

- Seven flood stations are 100% NULL in 2024–25 (dead sensors, reported to BMA); the serving network is therefore 100 stations.

- FW.PKG.01 shows flows to ±3,300 m³/s. Initially treated as a sensor fault, later re-identified via geocoding as the Rama VIII Bridge gauge on the Chao Phraya river — plausible river discharge with tidal reversal, but a different physical scale from canals; excluded from canal-scale features either way.

- A physically implausible 762 mm/24h rain spike (2023) and similar artifacts are clamped and flagged at ingestion.

- "Nonzero depth" is not "flood": early years show 1.4–3% nonzero label rates dominated by sub-5cm sensor noise. Labels are therefore defined on CAP depth tiers with a persistence rule, never on depth > 0.

## 2.3 Data pipeline

Phase 0 (data_pipeline/phase0_ingest.py): one streaming pass converts all 28 CSVs to typed, compressed Parquet (52 GB → ~8 GB), coercing NULLs, clamping implausible values, masking the river gauge, and emitting a per-station quality scorecard (3,736 station-years).

Training set (data_pipeline/build_training_set.py): one row per (flood station, 15-minute anchor). Rain features join per district — verified: every flood station's district-code prefix matches rain gauges (100% coverage). Water and flow enter as citywide aggregates pending sensor coordinates. Chronological splits: train 2019–23 (17.25M rows), validation 2024 (3.45M), test 2025 (3.35M, sealed).

# 3. The Model

## 3.1 What it predicts

Per flood station and per horizon (1h / 3h / 6h): (a) the probability that road flooding reaches each CAP tier — ≥5 cm, ≥15 cm, ≥30 cm — and (b) the flood-depth distribution as five quantiles (P05, P25, P50, P75, P95). Labels use strictly-future windows with a two-consecutive-readings persistence rule; windows with unobserved sensors are down-weighted via a validity score and never treated as "no flood".

## 3.2 Model ladder and selection

Models were added stepwise, each required to beat the previous on validation-2024 before advancing:

- Persistence baseline — "flooded now → flooded later". The honesty floor; strong at 1h, blind to incoming rain.

- LightGBM ensemble (SELECTED) — 9 binary classifiers (tier × horizon) + 15 quantile regressors (quantile × horizon). Class imbalance (0.03% positives at ≥15cm/1h) handled by negative downsampling (all positives + 5% of negatives) with a capped class weight; early stopping on validation PR-AUC. Probabilities are isotonic-calibrated before display.

- GRU dual-head (PyTorch, Apple-MPS) — one shared encoder over past-6h sequences with joint classification + quantile heads. Pre-registered keep rule: beat LightGBM validation PR-AUC on ge15@3h AND ge15@6h. Result: won 3h (0.195 vs 0.177), tied 6h (0.1015 vs 0.1018) → not kept. Notably it fixed the rare severe tier (ge30@1h PR-AUC 0.38 vs LightGBM's 0.0002), validating the shared-encoder hypothesis as the documented upgrade.

## 3.3 Serving policy (frozen on validation)

- Tiers ≥5cm and ≥15cm (all horizons) and ≥30cm@6h: hybrid rule — alert when model score ≥ validation threshold OR measured depth already ≥ tier.

- Tier ≥30cm at 1h/3h: sensor persistence only (too few severe events to learn standalone; F2 0.51/0.24 from persistence).

- Displayed risk % = isotonic-calibrated probability; raw scores are rankings only.

# 4. Inputs and Outputs

## 4.1 Inputs — 23 features + station identity

| Group | Features |
| --- | --- |
| Station's own history (7) | depth now; 1h/3h lags; 3h/24h rolling max; 1h mean; hours since ≥5cm |
| District rain (5) | rf1hr/rf3hr/rf24hr means, rf1hr max, 1h intensity change — joined by verified district prefix |
| Citywide canals (3) | mean 1h rise, share of stations rising, share offline |
| Citywide flow (3) | mean flow, share negative (tidal backflow), share offline |
| Calendar (5) | hour and day-of-year (sin/cos), monsoon flag (May–Oct) |
| Identity | station_code as categorical — lets the model learn each location's flooding temperament |

## 4.2 Outputs — the model→dashboard contract

Per station × horizon: calibrated risk % per tier, alert flags (serving policy applied), depth quantiles P05–P95, plus derived KPI fields the dashboard reads without recomputing: Peak Risk %, Peak Risk Time, Time-to-Warning, Peak P95 Depth, Time-to-15cm, Time-to-30cm. Area aggregation follows the supervisor rule: P95 across the district's stations is the primary displayed line, mean is secondary. Every response carries the station's district, latitude and longitude from the geocoded registry, and both endpoints emit GeoJSON on request for direct Leaflet mapping.

# 5. Results — sealed test on 2025

Thresholds and all decisions were frozen on validation-2024; the 2025 test year was evaluated exactly once (3.35M timesteps). Hybrid serving rule applied.

| Target | PR-AUC | Precision | Recall | F1 | F2 | Accuracy | Persist. F2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ≥5cm @1h | 0.584 | 0.330 | 0.609 | 0.428 | 0.521 | 99.87% | 0.579 |
| ≥15cm @1h | 0.560 | 0.644 | 0.548 | 0.592 | 0.565 | 99.98% | 0.553 |
| ≥30cm @1h * | 0.041 | 0.061 | 0.672 | 0.112 | 0.224 | 99.96% | 0.506 |
| ≥5cm @3h | 0.296 | 0.357 | 0.304 | 0.328 | 0.313 | 99.80% | 0.313 |
| ≥15cm @3h | 0.271 | 0.604 | 0.275 | 0.378 | 0.309 | 99.95% | 0.289 |
| ≥30cm @3h * | 0.010 | 0.031 | 0.326 | 0.056 | 0.111 | 99.91% | 0.245 |
| ≥5cm @6h | 0.181 | 0.343 | 0.175 | 0.232 | 0.194 | 99.68% | 0.187 |
| ≥15cm @6h | 0.160 | 0.676 | 0.154 | 0.251 | 0.183 | 99.92% | 0.170 |
| ≥30cm @6h | 0.073 | 0.300 | 0.116 | 0.167 | 0.132 | 99.98% | 0.138 |

- Headline: at the ≥15cm warning tier the model reaches 64% precision / 55% recall (F1 0.59) at 1h lead, and beats the persistence baseline on F2 at all three horizons.

- Accuracy exceeds 99.8% everywhere but is not meaningful at 0.03–0.3% positive rates — a model that never alerts scores identically. PR-AUC and F2 are the operative metrics.

- * ≥30cm at 1h/3h is served by persistence, not these classifiers (reported for completeness).

- Depth head: P95 and P05 pinball loss beat persistence at every horizon (e.g. q95@6h 0.0391 vs 0.0396) — the dashboard's P95 line is backed by a measured win. In the strongest caught event (FL.SMI.01, 2 Nov 2025), calibrated 1h risk read 80–93% and P95 depth 47.7–54.5 cm against an actual 46.4 cm flood.

- Error geography: misses concentrate at chronic-flood stations (FL.SMI.01, FL.BNA.04, FL.DDG.02). Notable false-negative class: rainless flash floods (e.g. FL.WTN.01, 8 Feb 2025 — 61.5 cm from dry with no district rain signal), invisible to any rain-driven model.

- What the model learned: at 1h, own-station recent depth (72% importance) plus rain intensification; at 6h, largely station identity and seasonality — i.e. climatology. This is the honest explanation of 6h recall and the reason live weather forecasts are the highest-leverage upgrade.

# 6. How the Outputs Are Delivered

| Endpoint | Purpose |
| --- | --- |
| /forecast?ts=…[&format=geojson] | Replay any 2025 timestamp: per-station contract, JSON or Leaflet-ready GeoJSON |
| /forecast/live[?format=geojson] | Live mode: real-time Open-Meteo rain per district; BMA sensor features marked offline (degraded, rain+climatology driven) |
| /forecast/area?ts=…&prefix=… | District aggregation: P95-primary risk line, mean secondary, stations alerting |
| /cap?ts=…&station=… | CAP 1.2 alert XML — severity from tier, urgency from horizon, certainty Observed/Likely, status always Test |
| /map | Built-in Leaflet map: live or replay, risk-colored markers, per-station popups |
| /docs, /health, /stations, /range | Interactive API docs and service metadata |

Station coordinates come from a 568-station geocoded registry built from the data itself (rain-station names contain district offices), public subdistrict gazetteers, and canal-name matching — 401 stations resolved at district/subdistrict-centroid precision (~1–2 km), with a documented confidence and basis per row. Flood stations (the mapped ones) are 100% covered. Good demo timestamps: 2025-11-13 03:00 (citywide storm, 25 stations flooding, 78 mm/h rain) versus 2025-01-01 00:00 (fully dry, all green); both windows are 100% sensor-valid.

# 7. Known Limitations

- No live weather forecast input yet — the 6h horizon is largely climatological (recall 15% at ≥15cm).

- Live mode runs without BMA sensor feeds: rain-driven only, persistence alerts cannot fire, short-horizon quality reduced. Alerts in live mode should key on the ≥15cm tier.

- ≥30cm tier not learnable standalone at short horizons from 7 years of data; served by persistence; GRU shared-encoder is the validated fix.

- Water/flow features are citywide aggregates until the remaining 167 canal-sensor coordinates are resolved (BMA's DDS portal has them but is unreachable outside Thailand; an OSM-based geocoding script is included).

- Probability calibration was fitted on a quiet validation year (2024) — recalibrate after the next monsoon.

- Universal 5/15/30 cm thresholds; per-station calibration is future work.

- Test-2025 is spent: any future model change requires a new holdout period.

# 8. Upgrade Roadmap

Each upgrade is independent, and none changes the output contract — the dashboard never needs to know.

| Upgrade | What it needs | What it buys |
| --- | --- | --- |
| Live BMA sensor feed | DDS telemetry access (weather.bangkok.go.th) | Live mode reaches replay quality; persistence alerts work in real time |
| Forecast rain input | Open-Meteo/TMD forecast API + retrain | Real predictive power at 6h beyond climatology — highest-leverage single change |
| Nearest-canal features | 167 remaining sensor coordinates | Replaces citywide aggregates with local canal state per station |
| GRU dual-head promotion | A fresh holdout year (2026) | Fixes severe-tier (≥30cm) short-horizon detection; single network replaces 24 models |
| Per-station thresholds | 2+ clean years per station | Alert calibration to each location's flooding temperament |
| Interpolated flood surface | Coordinates + DTM terrain (on hand) | Continuous risk map instead of point markers (IDW/Kriging) |
| Operational CAP dispatch | BMA alerting-channel integration | status=Actual alerts to real channels (currently Test-only by design) |

Artifact inventory: 24 LightGBM boosters, calibrators, frozen thresholds, GRU checkpoint and comparison, final metrics tables and per-station breakdowns (models/artifacts/); pipeline scripts and notebooks (data_pipeline/, notebooks/); serving code and map (backend/); station registry and quality scorecard (data/); companion documents dataset_research_report.md and model_report.md (docs/).